
 SEQ CHAPTER \h \r 10Chapter 1.10

FENCING STANDARDS

Sections: 

1.10.005 Fencing Standards.

1.10.010 Exceptions.

1.10.015 Clear view of intersecting streets.

1.10.020 Diagram.

1.10.005 FENCING STANDARDS.  The term “fence” shall include any tangible barrier, lattice work, screen, wall, hedge, or continuous growth of shrubs or trees with the purpose of, or having the effect of preventing passage or view across the fence line. Fences seven (7) feet or less in height shall require a no fee fence permit and approval by the zoning administrator. Fences seven (7) feet in height must obtain a building permit from the Building Inspection Department.

A.  Fences may be required. When approved by the governing body, the Land Use Authority may require the erection of fences as a prerequisite to the approval of any project or to the granting of any building permit where, in the opinion of said LUA, this is necessary to protect life and property. Such fences may be of the type and size necessary, in the above stated purpose.

B.  Front yard.  Solid walls, fences, hedges or screening materials which are sight obscuring may be built to a maximum of two (2) feet in any required front yard perimeter. Walls, fences, hedges or screening materials which are not sight obscuring ( at least 50% open ) may be built up to a maximum of four (4) feet in the front  yard.

  
C.  Side yard.  Solid, sight obscuring fences or walls may be built to a maximum of four (4) feet but may slope upward to connect with the higher rear yard fence. The slope length may not exceed one section or a maximum of ten (10) feet.

D.  Rear yard.  Walls and fences in a rear yard may exceed seven (7) feet provided a building permit is first obtained from the Building Inspection Department prior to construction.

E.  Corner lots.  A fence not more than six (6) feet high may be constructed in a side yard adjacent to a public street or neighboring property on a corner lot, provided it does not extend into the clear vision area of a corner lot as defined by section 1.01.310, or block adequate line of sight for driveways and alleys, this will be determined by the Zoning Administrator prior to issuing any permit.

1.10.010 EXCEPTIONS. The provisions of this section shall not apply to certain other fences such as tennis courts, backstops, or patio enclosures as approved by the Zoning Administrator if it is determined that such does not create a hazard or violation of other building codes or city ordinances.

A. For the purpose of this section, single shrub planting shall not constitute a hedge if the closest distance between the foliage of any two (2) plants is and remains at least five (5) feet.

B. Where a fence, wall, or hedge is located along a property line separating two (2) lots and there is a difference in grade of the properties on the two (2) sides of the property line, the fence, wall, or hedge may be erected or allowed to the maximum height permitted on either side of the property line.

C. Where sidewalk improvements have been made, fence lines may be adjacent to the sidewalk(s) in accordance with the provisions of this Ordinance.

1.10.015 CLEAR VIEW OF INTERSECTING STREETS (CORNER LOTS). In all districts which require a front yard(s), no obstruction to view in excess of two (2) feet in height shall be placed on any corner lot within a triangular area formed by the street property lines and a line connecting them at points not to exceed the setback of the home or building. Any fence, wall, screen, hedge, or other material serving as a fence, shall not create a sight distance hazard to vehicular or pedestrian traffic. See diagram 1.10.020.

(Am 01-07-97) (Am 01-18-00)(Am 10-19-04) (Am 10-16-07)
[image: image1.emf]
�





10.

10.


