[image: image1.jpg]SAFEbuiltic

COLORADO




REROOFING REQUIREMENTS
KEENESBURG COLORADO

Current Code 2006 International Codes
Contractor required to provide access with a safe secured ladder

No overlays permitted, all old material must be removed

Drip edge required on all edges including rakes
Ice and Water shield not required

All shingles must meet a manufactures wind speed of 90 MPH / Three tabs not permitted
Double underlayment for pitches of 2:12 up to 4:12

Dimensional shingles cannot be used for pitches under 2:12

Damaged or deteriorated vent caps or pipe / vent flashing must be replaced

Adequate attic ventilation shall be added if not existing
SAFEbuilt Colorado

11409 Business Park Circle Unit 350

Longmont, CO 80504

303-774-1388


